Hub75 synopsys project

Cell Generation & Testbench Comparison

September 24, 2024

Mark Ivey

UTD EECT 6325, VLSI Design

Design Vision Flipflop Output

Inferred memory devices in process

in routine hub75_g_hub_mux4_g_clock_freq200000000_g_color_depth8_g_num_row4_g_num_col4 line 107 in file

'/home/eng/m/mxi240020/6325/projects/hub75/project-1-library_pair/hub75.vhd'.

===============================================================================

|    Register Name    |   Type    | Width | Bus | MB | AR | AS | SR | SS | ST |

===============================================================================

|   hub_blank_o_reg   | Flip-flop |   1   |  N  | N  | N  | N  | N  | N  | N  |

|   hub_latch_o_reg   | Flip-flop |   1   |  N  | N  | N  | N  | N  | N  | N  |

|    hub_clk_o_reg    | Flip-flop |   1   |  N  | N  | N  | N  | N  | N  | N  |

|    hub_mux_o_reg    | Flip-flop |   4   |  Y  | N  | N  | N  | N  | N  | N  |

|    bcd_time_reg     | Flip-flop |   3   |  Y  | N  | N  | N  | N  | N  | N  |

|      h_row_reg      | Flip-flop |  32   |  Y  | N  | N  | N  | N  | N  | N  |

|      h_col_reg      | Flip-flop |  32   |  Y  | N  | N  | N  | N  | N  | N  |

|   disp_timer_reg    | Flip-flop |  32   |  Y  | N  | N  | N  | N  | N  | N  |

|    clk_timer_reg    | Flip-flop |  32   |  Y  | N  | N  | N  | N  | N  | N  |

|    hub_reg_f_reg    | Flip-flop |   1   |  N  | N  | N  | N  | N  | N  | N  |

|    hub_mem_f_reg    | Flip-flop |   1   |  N  | N  | N  | N  | N  | N  | N  |

|   hub_out_sm_reg    | Flip-flop |   3   |  Y  | N  | N  | N  | N  | N  | N  |

|   hub_piso_r0_reg   | Flip-flop |   8   |  Y  | N  | N  | N  | N  | N  | N  |

|   hub_piso_g0_reg   | Flip-flop |   8   |  Y  | N  | N  | N  | N  | N  | N  |

|   hub_piso_b0_reg   | Flip-flop |   8   |  Y  | N  | N  | N  | N  | N  | N  |

|    hub_r0_o_reg     | Flip-flop |   1   |  N  | N  | N  | N  | N  | N  | N  |

|    hub_g0_o_reg     | Flip-flop |   1   |  N  | N  | N  | N  | N  | N  | N  |

|    hub_b0_o_reg     | Flip-flop |   1   |  N  | N  | N  | N  | N  | N  | N  |

===============================================================================

Presto compilation completed successfully.

Information: Building the design 'bram_xfer' instantiated from design 'input_mem_g_color_depth8_g_num_row4_g_num_col4' with

the parameters "g_addr_width=4,g_color_depth=8". (HDL-193)

Inferred memory devices in process

in routine bram_xfer_g_addr_width4_g_color_depth8 line 29 in file

'/home/eng/m/mxi240020/6325/projects/hub75/project-1-library_pair/bram_xfer.vhd'.

===============================================================================

|    Register Name    |   Type    | Width | Bus | MB | AR | AS | SR | SS | ST |

===============================================================================

|   data_out_o_reg    | Flip-flop |   8   |  Y  | N  | N  | N  | N  | N  | N  |

|   block_ram_r_reg   | Flip-flop |  256  |  Y  | N  | N  | N  | N  | N  | N  |

===============================================================================

Last Lines of the Cell Report:

registers/s_mem_sipo_r0_reg[7]                                                           						                 dff             library         7.000000  n

--------------------------------------------------------------------------------

Total 4321 cells                                          11668.000000

All code can be found here: https://github.com/Mivey/HUB75_Fork

bram_xfer.vhd file

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use ieee.math_real.all;

entity bram_xfer is

generic (

g_addr_width : integer := 8;

g_color_depth : integer := 8

);

port (

clk        : in    std_logic;

en         : in    std_logic;

we_i       : in    std_logic;

addr_i     : in    unsigned(g_addr_width - 1 downto 0);

data_in_i  : in    std_logic_vector(g_color_depth - 1 downto 0);

data_out_o : out   std_logic_vector(g_color_depth - 1 downto 0)

);

end entity bram_xfer;

architecture rtl of bram_xfer is

type t_ram is array(2 ** g_addr_width - 1 downto 0) of std_logic_vector(g_color_depth - 1 downto 0);

signal block_ram_r : t_ram;

begin

ram_operation : process (clk) is

begin

if rising_edge(clk) then

if (en = '1') then

if (we_i = '1') then

block_ram_r (to_integer(addr_i)) <= data_in_i;

data_out_o <= data_in_i;

else

data_out_o <= block_ram_r(to_integer(addr_i));

end if;

end if;

end if;

end process ram_operation;

end architecture rtl;

hub75.vhd file

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use ieee.math_real.all;

entity hub75 is

generic (

g_hub_mux     : integer := 4;

g_clock_freq  : integer := 200_000_000;

g_color_depth : integer := 8;

g_num_row     : integer := 4;

g_num_col     : integer := 4

);

port (

clk         : in    std_logic;

hub_mux_o   : out   unsigned(g_hub_mux - 1 downto 0);

hub_clk_o   : out   std_logic;

hub_latch_o : out   std_logic;

hub_blank_o : out   std_logic;

hub_g0_o    : out   std_logic;

hub_b0_o    : out   std_logic;

hub_r0_o    : out   std_logic;

mem_sipo_r0 : in    bit_vector(g_color_depth - 1 downto 0);

mem_sipo_g0 : in    bit_vector(g_color_depth - 1 downto 0);

mem_sipo_b0 : in    bit_vector(g_color_depth - 1 downto 0);

hub_reg_f   : out   bit;

hub_mem_f   : out   bit;

mem_hub_f   : in    bit;

reg_hub_f   : in    bit

);

end entity hub75;

architecture rtl of hub75 is

type hub_sm is (sm_reset, sm_clk_low, sm_clk_hi, sm_delay, sm_latch_hi);

signal hub_out_sm : hub_sm := sm_reset;

constant hub_clk_tick : integer := (g_clock_freq / 20_000_000) - 1;

type tick_array is array (natural range<>) of integer;

constant disp_tick : tick_array(g_color_depth - 1 downto 0) :=

( 13056 / (2 ** 7) - 1, 13088 / (2 ** 6) - 1, 13088 / (2 ** 5) - 1,

13088 / (2 ** 4) - 1, 13088 / (2 ** 3) - 1, 13088 / (2 ** 2) - 1,

13088 / (2 ** 1) - 1, 13088 / (2 ** 0) – 1 );

signal h_row       : integer;

signal h_col       : integer;

signal disp_timer  : integer;

signal clk_timer   : integer;

signal bcd_time    : integer range 0 to g_color_depth - 1;

signal hub_piso_r0 : bit_vector(g_color_depth - 1 downto 0);

signal hub_piso_g0 : bit_vector(g_color_depth - 1 downto 0);

signal hub_piso_b0 : bit_vector(g_color_depth - 1 downto 0);

begin

hub_state_machine : process (clk) is

begin

if rising_edge(clk) then

case hub_out_sm is

-----------------------reset----------------------

when sm_reset =>

hub_blank_o <= '1';

hub_latch_o <= '0';

hub_clk_o   <= '0';

hub_mux_o   <= (others => '1');

bcd_time    <= 0;

h_row       <= 0;

h_col       <= 0;

disp_timer  <= disp_tick(0);

clk_timer   <= hub_clk_tick;

hub_reg_f   <= '0';

hub_mem_f   <= '1';

if (mem_hub_f = '1') then

if (reg_hub_f = '1') then -- do I need reg_comp_f?

hub_piso_r0 <= mem_sipo_r0;

hub_piso_g0 <= mem_sipo_g0;

hub_piso_b0 <= mem_sipo_b0;

hub_out_sm  <= sm_clk_low;

hub_reg_f   <= '1';

elsif (reg_hub_f = '0') then

hub_reg_f  <= '0';

hub_out_sm <= sm_reset;

end if;

end if;

----------------------- clk low----------------------

when sm_clk_low =>

hub_r0_o <= to_stdulogic(hub_piso_r0(0));

hub_g0_o <= to_stdulogic(hub_piso_g0(0));

hub_b0_o <= to_stdulogic(hub_piso_b0(0));

disp_timer <= disp_timer - 1;

clk_timer  <= clk_timer - 1;

if (clk_timer <= (hub_clk_tick / 2)) then

hub_out_sm <= sm_clk_hi;

hub_clk_o  <= '1';

elsif (clk_timer > (hub_clk_tick / 2)) then

hub_out_sm <= sm_clk_low;

hub_reg_f  <= '0';

hub_clk_o  <= '0';

end if;

----------------------- clk high----------------------

when sm_clk_hi =>

if (clk_timer = 0) then

clk_timer  <= hub_clk_tick;                             -- reset timer

disp_timer <= disp_timer - 1;

hub_clk_o  <= '0';

if (h_col < g_num_col - 1) then

hub_out_sm <= sm_clk_low;                     -- go back and send next bit

h_col      <= h_col + 1;

for i in 0 to g_color_depth - 2 loop

hub_piso_r0(i) <= hub_piso_r0(i + 1);

hub_piso_g0(i) <= hub_piso_g0(i + 1);

hub_piso_b0(i) <= hub_piso_b0(i + 1);

end loop;

else                                         -- just sent the MSB, so now wait

hub_out_sm <= sm_delay;

end if;

elsif (clk_timer > 0) then

disp_timer <= disp_timer - 1;

clk_timer  <= clk_timer - 1;

hub_out_sm <= sm_clk_hi;

hub_clk_o  <= '1';

end if;

----------------------- timer delay----------------------

when sm_delay =>

hub_clk_o <= '0';

if (disp_timer > 0) then

disp_timer <= disp_timer - 1;

hub_out_sm <= sm_delay;

elsif (disp_timer = 0) then

bcd_time    <= bcd_time - 1;

disp_timer  <= disp_tick(bcd_time - 1);

clk_timer   <= 20;

hub_latch_o <= '1';

hub_out_sm  <= sm_latch_hi;

hub_piso_r0 <= mem_sipo_r0;

hub_piso_g0 <= mem_sipo_g0;

hub_piso_b0 <= mem_sipo_b0;

if (bcd_time = g_color_depth - 1) then

h_row       <= h_row + 1;

hub_blank_o <= '1';

hub_mux_o   <= to_unsigned (h_row, hub_mux_o'length);

end if;

end if;

----------------------- latch high----------------------

when sm_latch_hi =>

disp_timer <= disp_timer - 1;

if (clk_timer > 0) then

clk_timer   <= clk_timer - 1;

hub_out_sm  <= sm_latch_hi;

hub_latch_o <= '1';

elsif (clk_timer = 0) then

if (mem_hub_f = '0') then

hub_out_sm <= sm_reset;

elsif (reg_hub_f = '0') then

hub_out_sm <= sm_latch_hi;

else

hub_out_sm <= sm_clk_low;

hub_reg_f <= '1';

end if;

hub_latch_o <= '0';

hub_blank_o <= '0';

end if;

when others =>

null;

end case;

end if;

end process hub_state_machine;

end architecture rtl;

input_mem.vhd file

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use ieee.math_real.all;

entity input_mem is

generic (

g_color_depth : integer := 8;

g_num_row     : integer := 8;

g_num_col     : integer := 8

);

port (

clk         : in    std_logic;

reset       : in    bit;

top_data    : in    std_logic_vector(24 - 1 downto 0);

top_dv_i    : in    std_logic;

top_u_l     : in    std_logic;

top_ack     : out   std_logic;

mem_out_r0  : out   std_logic_vector(g_color_depth - 1 downto 0);

mem_out_g0  : out   std_logic_vector(g_color_depth - 1 downto 0);

mem_out_b0  : out   std_logic_vector(g_color_depth - 1 downto 0);

addr_i      : in    unsigned(4 - 1 downto 0);

mem_hub_f   : out   bit;

hub_mem_f   : in    bit;

reg_mem_f   : in    bit;

mem_reg_f   : out   bit

);

end entity input_mem;

architecture rtl of input_mem is

component bram_xfer

generic (

g_addr_width : integer;

g_color_depth : integer

);

port (

clk : in std_logic;

en : in std_logic;

we_i : in std_logic;

addr_i : in unsigned(g_addr_width - 1 downto 0);

data_in_i : in std_logic_vector(g_color_depth - 1 downto 0);

data_out_o : out std_logic_vector(g_color_depth - 1 downto 0)

);

end component;

type mem_sm is (sm_load, sm_complete, sm_read);

signal simple_mem_sm : mem_sm := sm_complete;

type temp_mem is array (0 to 2) of std_logic_vector(g_color_depth - 1 downto 0);

signal lazy_out : temp_mem;

signal lazy_in : temp_mem;

signal mem_timer   : integer;

signal we_i : std_logic;

signal s_addr : unsigned(addr_i'range);

signal mem_full : bit;

begin

RGB_Array : for i in lazy_out'range generate

RGB_Array: bram_xfer

generic map (

g_addr_width => 4,

g_color_depth => g_color_depth

)

port map (

clk   => clk,

en => '1',

we_i => we_i,

addr_i => addr_i,

data_in_i => lazy_in(i),

data_out_o => lazy_out(i)

);

end generate;

mem_loader : process (clk) is

begin

---- this process needs to see that both reg and hub are in a state of reset before it releases its own reset.

if rising_edge(clk) then

if (reset = '0') then

simple_mem_sm <= sm_complete;

mem_reg_f     <= '0';

mem_hub_f     <= '0';

we_i          <= '0';

elsif (reset = '1') then

case simple_mem_sm is

when sm_load =>

if (top_dv_i = '0' or (mem_full = '1')) then

top_ack       <= '1';

simple_mem_sm <= sm_complete;

we_i <= '0';

mem_timer <= 0;

else

lazy_in(2) <= top_data(3 * g_color_depth - 1 downto 2 * g_color_depth);

lazy_in(1) <= top_data(2 * g_color_depth - 1 downto 1 * g_color_depth);

lazy_in(0) <= top_data(1 * g_color_depth - 1 downto 0 * g_color_depth);

if (s_addr = "111111") then

mem_full <= '1';

else

mem_full <= '0';

end if;

s_addr <= s_addr + 1;

simple_mem_sm <= sm_load;

end if;

when sm_complete =>

if (top_dv_i = '1') then

if (mem_timer < 7) then

simple_mem_sm <= sm_complete;

mem_timer <= mem_timer + 1;

elsif (mem_timer >= 7) then

simple_mem_sm <= sm_load;

mem_full <= '0';

s_addr <= (others => '0');

mem_timer <= 0;

top_ack <= '0';

mem_hub_f <= '0';

mem_reg_f <= '0';

we_i <= '1';

end if;

elsif (top_dv_i = '0') then

mem_timer <= 0;

mem_hub_f <= '1';

mem_reg_f <= '1';

simple_mem_sm <= sm_read;

s_addr <= addr_i;

we_i <= '0';

end if;

when sm_read =>

if (top_dv_i = '0') then

s_addr <= addr_i;

mem_out_r0 <= lazy_out(2);

mem_out_g0 <= lazy_out(1);

mem_out_b0 <= lazy_out(0);

mem_hub_f <= '1';

mem_reg_f <= '1';

elsif (top_dv_i = '1') then

simple_mem_sm <= sm_complete;

mem_hub_f <= '0';

mem_reg_f <= '0';

end if;

when others =>

null;

end case;

end if;

end if;

end process mem_loader;

end architecture rtl;

sipo_reg.vhd file

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use ieee.math_real.all;

entity sipo_reg is

generic (

g_color_depth : integer := 8;

g_num_row     : integer := 4;

g_num_col     : integer := 4

);

port (

clk         : in    std_logic;

mem_sipo_r0 : out   bit_vector(g_color_depth - 1 downto 0);

mem_sipo_g0 : out   bit_vector(g_color_depth - 1 downto 0);

mem_sipo_b0 : out   bit_vector(g_color_depth - 1 downto 0);

mem_out_r0  : in    std_logic_vector(g_color_depth - 1 downto 0);

mem_out_g0  : in    std_logic_vector(g_color_depth - 1 downto 0);

mem_out_b0  : in    std_logic_vector(g_color_depth - 1 downto 0);

addr_i      : out   unsigned(4 - 1 downto 0);

reg_hub_f   : out   bit;

hub_reg_f   : in    bit;

reg_mem_f   : out   bit;

mem_reg_f   : in    bit

);

end entity sipo_reg;

architecture rtl of sipo_reg is

type reg_sm is (sm_stop, sm_load, sm_done);

signal mem_to_reg : reg_sm := sm_stop;

signal reg_col_inc   : integer range 0 to g_num_col - 1;

signal reg_row       : integer range 0 to g_num_row - 1;

signal reg_cd        : integer range 0 to g_color_depth - 1;

signal s_mem_sipo_r0 : bit_vector(g_color_depth - 1 downto 0);

signal s_mem_sipo_g0 : bit_vector(g_color_depth - 1 downto 0);

signal s_mem_sipo_b0 : bit_vector(g_color_depth - 1 downto 0);

signal s_delay       : unsigned(1 downto 0);

begin

reg_loader : process (clk) is

begin

if rising_edge(clk) then

case mem_to_reg is

when sm_stop =>

reg_hub_f <= '0';

if (mem_reg_f = '1') then

mem_to_reg  <= sm_load;

reg_row     <= 0;

reg_cd      <= 0;

reg_col_inc <= 0;

addr_i      <= (others => '0');

addr_i <= to_unsigned(reg_row, 2) & to_unsigned(reg_col_inc, 2);

elsif (mem_reg_f <= '0') then

mem_to_reg <= sm_stop;

end if;

when sm_load =>

if (mem_reg_f = '1') then

for i in g_color_depth - 2 downto 0 loop

s_mem_sipo_r0(i) <= s_mem_sipo_r0(i + 1);

s_mem_sipo_g0(i) <= s_mem_sipo_g0(i + 1);

s_mem_sipo_b0(i) <= s_mem_sipo_b0(i + 1);

end loop;

s_mem_sipo_r0(7) <= to_bit(mem_out_r0(reg_cd));

s_mem_sipo_g0(7) <= to_bit(mem_out_g0(reg_cd));

s_mem_sipo_b0(7) <= to_bit(mem_out_b0(reg_cd));

addr_i <= to_unsigned(reg_row, 2) & to_unsigned(reg_col_inc, 2);

if (reg_col_inc < g_num_col - 1) then

mem_to_reg  <= sm_load;

reg_hub_f   <= '0';

reg_col_inc <= reg_col_inc + 1;

elsif (reg_col_inc >= g_num_col - 1) then

mem_to_reg  <= sm_done;

s_delay <= "11";

reg_hub_f   <= '0';

reg_col_inc <= 0;

if (reg_cd < g_color_depth - 1) then

reg_cd <= reg_cd + 1;

elsif (reg_cd >= g_color_depth - 1) then

reg_cd  <= 0;

if (reg_row < g_num_row - 1) then

reg_row <= reg_row + 1;

elsif (reg_row >= g_num_row - 1) then

reg_row <= 0;

end if;

end if;

end if;

elsif (mem_reg_f = '0') then

mem_to_reg  <= sm_stop;

reg_hub_f   <= '0';

reg_col_inc <= 0;

end if;

when sm_done =>

mem_sipo_r0 <= s_mem_sipo_r0;

mem_sipo_g0 <= s_mem_sipo_g0;

mem_sipo_b0 <= s_mem_sipo_b0;

if (s_delay = "00") then

if (mem_reg_f = '1') then

if (hub_reg_f = '0') then

mem_to_reg <= sm_done;

reg_hub_f  <= '1';

elsif (hub_reg_f = '1') then

mem_to_reg <= sm_load;

reg_hub_f  <= '0';

end if;

elsif (mem_reg_f = '0') then

mem_to_reg <= sm_stop;

reg_hub_f  <= '0';

end if;

else

s_delay <= s_delay - 1;

reg_hub_f   <= '1';

end if;

when others =>

null;

end case;

end if;

end process reg_loader;

end architecture rtl;

top.vhd file

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use ieee.math_real.all;

entity top is

generic (

g_hub_mux     : integer := 4;

g_clock_freq  : integer := 200_000_000;

g_color_depth : integer := 8;

g_num_row     : integer := 4;

g_num_col     : integer := 4

);

port (

clk         : in    std_logic;

reset       : in    bit;

top_data    : in    std_logic_vector(24 - 1 downto 0);

top_dv_i    : in    std_logic;

top_u_l     : in    std_logic;

top_ack     : out   std_logic;

hub_mux_o   : out   unsigned(g_hub_mux - 1 downto 0);

hub_clk_o   : out   std_logic;

hub_latch_o : out   std_logic;

hub_blank_o : out   std_logic;

hub_g0_o    : out   std_logic;

hub_b0_o    : out   std_logic;

hub_r0_o    : out   std_logic

);

end entity top;

architecture rtl of top is

component hub75

generic (

g_hub_mux     : integer;

g_clock_freq  : integer;

g_color_depth : integer;

g_num_row     : integer;

g_num_col     : integer

);

port (

clk         : in    std_logic;

hub_mux_o   : out   unsigned(g_hub_mux - 1 downto 0);

hub_clk_o   : out   std_logic;

hub_latch_o : out   std_logic;

hub_blank_o : out   std_logic;

hub_g0_o    : out   std_logic;

hub_b0_o    : out   std_logic;

hub_r0_o    : out   std_logic;

mem_sipo_r0 : in    bit_vector(g_color_depth - 1 downto 0);

mem_sipo_g0 : in    bit_vector(g_color_depth - 1 downto 0);

mem_sipo_b0 : in    bit_vector(g_color_depth - 1 downto 0);

hub_reg_f   : out   bit;

hub_mem_f   : out   bit;

mem_hub_f   : in    bit;

reg_hub_f   : in    bit

);

end component;

component sipo_reg

generic (

g_color_depth : integer;

g_num_row     : integer;

g_num_col     : integer

);

port (

clk         : in    std_logic;

mem_sipo_r0 : out   bit_vector(g_color_depth - 1 downto 0);

mem_sipo_g0 : out   bit_vector(g_color_depth - 1 downto 0);

mem_sipo_b0 : out   bit_vector(g_color_depth - 1 downto 0);

mem_out_r0  : in    std_logic_vector(g_color_depth - 1 downto 0);

mem_out_g0  : in    std_logic_vector(g_color_depth - 1 downto 0);

mem_out_b0  : in    std_logic_vector(g_color_depth - 1 downto 0);

addr_i      : out   unsigned(4 - 1 downto 0);

reg_hub_f   : out   bit;

hub_reg_f   : in    bit;

reg_mem_f   : out   bit;

mem_reg_f   : in    bit

);

end component;

component input_mem

generic (

g_color_depth : integer;

g_num_row     : integer;

g_num_col     : integer

);

port (

clk        : in    std_logic;

reset      : in    bit;

top_data   : in    std_logic_vector(24 - 1 downto 0);

top_dv_i   : in    std_logic;

top_u_l    : in    std_logic;

top_ack    : out   std_logic;

mem_out_r0 : out   std_logic_vector(g_color_depth - 1 downto 0);

mem_out_g0 : out   std_logic_vector(g_color_depth - 1 downto 0);

mem_out_b0 : out   std_logic_vector(g_color_depth - 1 downto 0);

addr_i     : in    unsigned(4 - 1 downto 0);

mem_hub_f  : out   bit;

hub_mem_f  : in    bit;

reg_mem_f  : in    bit;

mem_reg_f  : out   bit

);

end component;

signal mem_sipo_r0 :  bit_vector(g_color_depth - 1 downto 0);

signal mem_sipo_g0 :  bit_vector(g_color_depth - 1 downto 0);

signal mem_sipo_b0 :  bit_vector(g_color_depth - 1 downto 0);

signal reg_hub_f   :  bit;

signal hub_reg_f   :  bit;

signal mem_out_r0  :   std_logic_vector(g_color_depth - 1 downto 0);

signal mem_out_g0  :   std_logic_vector(g_color_depth - 1 downto 0);

signal mem_out_b0  :   std_logic_vector(g_color_depth - 1 downto 0);

signal addr_i      :   unsigned(4 - 1 downto 0);

signal mem_hub_f   :   bit;

signal hub_mem_f   :   bit;

signal reg_mem_f   :   bit;

signal mem_reg_f   :   bit;

begin

registers: sipo_reg

generic map (

g_color_depth => g_color_depth,

g_num_row     => g_num_row,

g_num_col     => g_num_col

)

port map (

clk   => clk,

mem_sipo_r0 => mem_sipo_r0, mem_sipo_g0 => mem_sipo_g0, mem_sipo_b0 => mem_sipo_b0,

mem_out_r0  => mem_out_r0, mem_out_g0  => mem_out_g0, mem_out_b0  => mem_out_b0,

addr_i => addr_i, reg_hub_f => reg_hub_f, hub_reg_f => hub_reg_f,

reg_mem_f => reg_mem_f, mem_reg_f => mem_reg_f

);

load_n_store: input_mem

generic map (

g_color_depth => g_color_depth,

g_num_row     => g_num_row,

g_num_col     => g_num_col

)

port map (

clk        => clk,

reset      => reset,

top_data   => top_data,

top_dv_i   => top_dv_i,

top_u_l    => top_u_l,

top_ack    => top_ack,

mem_out_r0 => mem_out_r0,

mem_out_g0 => mem_out_g0,

mem_out_b0 => mem_out_b0,

addr_i     => addr_i,

mem_hub_f  => mem_hub_f,

hub_mem_f  => hub_mem_f,

reg_mem_f  => reg_mem_f,

mem_reg_f  => mem_reg_f

);

display_out: hub75

generic map (

g_hub_mux     => g_hub_mux,

g_clock_freq  => g_clock_freq,

g_color_depth => g_color_depth,

g_num_row     => g_num_row,

g_num_col     => g_num_col

)

port map (

clk   => clk,

hub_mux_o   =>  hub_mux_o,

hub_clk_o   =>  hub_clk_o,

hub_latch_o =>  hub_latch_o,

hub_blank_o =>  hub_blank_o,

hub_g0_o    =>  hub_g0_o,

hub_b0_o    =>  hub_b0_o,

hub_r0_o    =>  hub_r0_o,

mem_sipo_r0 =>  mem_sipo_r0,

mem_sipo_g0 =>  mem_sipo_g0,

mem_sipo_b0 =>  mem_sipo_b0,

hub_reg_f   =>  hub_reg_f,

hub_mem_f   =>  hub_mem_f,

mem_hub_f   =>  mem_hub_f,

reg_hub_f   =>  reg_hub_f

);

end architecture;

Top_tb.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity top_tb is

end;

architecture bench of top_tb is

-- Clock period

constant clk_period : time := 5 ns;

-- Generics

constant g_hub_mux : integer := 3;

constant g_clock_freq : integer := 200_000_000;

constant g_color_depth : integer := 8;

constant g_num_row : integer := 4;

constant g_num_col : integer := 8;

constant g_u_row  : integer := 2;

constant g_u_col  : integer := 3;

constant g_addr_width  : integer := 5;

-- Ports

signal clk : std_logic;

signal reset : bit;

signal top_data : std_logic_vector(24 - 1 downto 0);

signal top_dv_i : std_logic;

signal top_u_l : std_logic;

signal top_ack : std_logic;

signal hub_mux_o : unsigned(g_hub_mux - 1 downto 0);

signal hub_clk_o : std_logic;

signal hub_latch_o : std_logic;

signal hub_blank_o : std_logic;

signal hub_g0_o : std_logic;

signal hub_b0_o : std_logic;

signal hub_r0_o : std_logic;

signal finished : boolean := false;

begin

top_inst : entity work.top

generic map (

g_hub_mux => g_hub_mux,

g_clock_freq => g_clock_freq,

g_color_depth => g_color_depth,

g_num_row => g_num_row,

g_num_col => g_num_col,

g_u_row => g_u_row,

g_u_col => g_u_col,

g_addr_width => g_addr_width

)

port map (

clk => clk,

reset => reset,

top_data => top_data,

top_dv_i => top_dv_i,

top_u_l => top_u_l,

top_ack => top_ack,

hub_mux_o => hub_mux_o,

hub_clk_o => hub_clk_o,

hub_latch_o => hub_latch_o,

hub_blank_o => hub_blank_o,

hub_g0_o => hub_g0_o,

hub_b0_o => hub_b0_o,

hub_r0_o => hub_r0_o

);

process is

begin

clk <= '1';

wait for clk_period;

clk <= '0';

wait for clk_period;

if finished then

wait;

end if;

end process;

process is

variable temp : unsigned(3 downto 0);

begin

reset <= '0';

wait for clk_period * 10;

reset <= '1';

wait for clk_period * 2;

top_dv_i <= '1';

top_data <= X"AA55AA";

wait for clk_period * 16;

for i in 0 to 7 loop

temp := to_unsigned(i, 4);

if (temp(0) = '0' ) then

wait for clk_period * 2;

top_data <= X"55AA55";

else

wait for clk_period * 2;

top_data <= X"AA55AA";

end if;

end loop;

wait for clk_period * 2;

top_data <= X"dec0de";

wait for clk_period * 2;

top_data <= X"c0ffee";

for i in 0 to 7 loop

temp := to_unsigned(i, 4);

if (temp(0) = '0' ) then

wait for clk_period * 2;

top_data <= X"55AA55";

else

wait for clk_period * 2;

top_data <= X"AA55AA";

end if;

-- wait for clk_period * 2;

end loop;

wait for clk_period * 2;

top_dv_i <= '0';

wait on top_ack for 1000 * clk_period;

wait for clk_period * 20;

for i in 0 to 31 loop

-- reg_hub_f <= '0';

wait for clk_period * 120880;

end loop;

for i in 0 to 31 loop

wait for 2 * clk_period;

end loop;

wait for 2 * clk_period;

wait for clk_period * 2;

top_dv_i <= '1';

top_data <= X"AA55AA";

wait for clk_period * 16;

for i in 0 to 7 loop

temp := to_unsigned(i, 4);

if (temp(0) = '0' ) then

wait for clk_period * 2;

top_data <= X"55AA55";

else

wait for clk_period * 2;

top_data <= X"AA55AA";

end if;

-- wait for clk_period * 2;

end loop;

wait for clk_period * 2;

top_data <= X"beef1e";

wait for clk_period * 2;

top_data <= X"b0d1e5";

for i in 0 to 7 loop

temp := to_unsigned(i, 4);

if (temp(0) = '1' ) then

wait for clk_period * 2;

top_data <= X"a5a5a5";

else

wait for clk_period * 2;

top_data <= X"969696";

end if;

-- wait for clk_period * 2;

end loop;

wait for clk_period * 2;

top_dv_i <= '0';

wait on top_ack for 1000 * clk_period;

wait for clk_period * 20;

for i in 0 to 31 loop

wait for 2 * clk_period;

end loop;

wait for 2 * clk_period;

finished <= true;

wait for clk_period * 2;

wait;

end process;

end;

Figure 1: Identical full panel output from behavioral and Synopsys netlist.

Figure 2: Output of one row of LED's, behavioral vs netlist.

Figure 3: Zoom in on data transfer, Binary Coded Modulation currently being displayed. The wait time between clock and data activity is halved as expected.

Figure 4: Zoom in on one clock and data transfer.