Due Date: Sunday September 22, 11:59pm – Submit this editable Word document NOT pdf

su

0

16

EEDG/CE 6370

Design and Analysis of Reconfigurable Systems

Homework 3 – IP generator

Name: Mark Ivey

Email: mxi240020@utdallas.edu

Part I – CORDIC IP

a.) Follow the instructions in the lab sheet. Show the simulation result of the CORDIC IP clearly indicating that the circuit works

Marks

4

b.) Report the number of ALMs and critical path of the design from the synthesis report. Compute the maximum frequency.

Marks

2

Fmax = 1/14.171 ns = 70.566 MHz

c.) Create a short video showing that the system is successfully prototyped on the DE1-SoC Board

Marks

2

YouTube Link

Part II – CORDIC IP Output Modification (7-segment display)

- Modify the program such that the value of the sin OR cosine are output on the 7-segment display instead of the LEDs.

- The indicated push-button will toggle between displaying the sine or cosine

- As shown below the first 7-segment display should show the sign (+ or -) and the rest  should display the value

Cosine OR sine

value [+1,-1]

sign

Include here your code and a short video showing that it works. Report also the number of resources used and the critical path

Marks

8

YouTube Link

Delay was 13.773 ns

Fmax = 72.606 MHz

Sincos_disp_top.vhd

library ieee;

use ieee.std_logic_1164.all;

entity sincos_disp_top is

port (

clk   : in std_logic;

reset : in std_logic;

cd_button   : in std_logic;

cordic_data_s : in std_logic_vector(4 downto 0);

cordic_data_c : in std_logic_vector(4 downto 0);

sev_seg_0_o : out std_logic_vector(6 downto 0);

sev_seg_1_o : out std_logic_vector(6 downto 0);

sev_seg_2_o : out std_logic_vector(6 downto 0);

sev_seg_3_o : out std_logic_vector(6 downto 0);

sev_seg_4_o : out std_logic_vector(6 downto 0);

sev_seg_5_o : out std_logic_vector(6 downto 0)

);

end entity;

architecture rtl of sincos_disp_top is

component seven_seg

port (

data : in std_logic_vector(4 downto 0);

led : out std_logic_vector(6 downto 0)

);

end component;

signal s_sev_seg_0_data : std_logic_vector(4 downto 0);

signal s_sev_seg_1_data : std_logic_vector(4 downto 0);

signal s_sev_seg_2_data : std_logic_vector(4 downto 0);

signal s_sev_seg_3_data : std_logic_vector(4 downto 0);

signal s_sev_seg_4_data : std_logic_vector(4 downto 0);

signal s_sev_seg_5_data : std_logic_vector(4 downto 0);

signal ip_data : std_logic_vector(4 downto 0);

begin

seven_seg_0_inst : entity work.seven_seg

port map (

data => s_sev_seg_0_data,

led => sev_seg_0_o

);

seven_seg_1_inst : entity work.seven_seg

port map (

data => s_sev_seg_1_data,

led => sev_seg_1_o

);

seven_seg_2_inst : entity work.seven_seg

port map (

data => s_sev_seg_2_data,

led => sev_seg_2_o

);

seven_seg_3_inst : entity work.seven_seg

port map (

data => s_sev_seg_3_data,

led => sev_seg_3_o

);

seven_seg_4_inst : entity work.seven_seg

port map (

data => s_sev_seg_4_data,

led => sev_seg_4_o

);

seven_seg_5_inst : entity work.seven_seg

port map (

data => s_sev_seg_5_data,

led => sev_seg_5_o

);

-- in keeping with the spirit of avoiding multiplication

-- this case statement will map all 16 possibilities

-- "if foo == 1; Printf “odd!”, if foo == 2; printf “even!”, etc"

process (clk)

begin

if rising_edge(clk) then

if (reset /= '0') then

s_sev_seg_5_data <= "01001";

s_sev_seg_4_data <= "11110";

s_sev_seg_3_data <= "00100";

s_sev_seg_2_data <= "11110";

s_sev_seg_1_data <= "01010";

s_sev_seg_0_data <= "00000";

elsif (reset = '0') then

if (cd_button = '1') then

ip_data <= cordic_data_c;

else

ip_data <= cordic_data_s;

end if;

case ip_data is

when "11000" => -- -1

s_sev_seg_5_data <= "01000";

s_sev_seg_4_data <= "10001";

s_sev_seg_3_data <= "00001";

s_sev_seg_2_data <= "10000";

s_sev_seg_1_data <= "10000";

s_sev_seg_0_data <= "10000";

when "11001" => -- -0.875

s_sev_seg_5_data <= "01000";

s_sev_seg_4_data <= "10000";

s_sev_seg_3_data <= "00001";

s_sev_seg_2_data <= "11000";

s_sev_seg_1_data <= "10111";

s_sev_seg_0_data <= "10101";

when "11010" => -- -0.750

s_sev_seg_5_data <= "01000";

s_sev_seg_4_data <= "10000";

s_sev_seg_3_data <= "00001";

s_sev_seg_2_data <= "10111";

s_sev_seg_1_data <= "10101";

s_sev_seg_0_data <= "10000";

when "11011" => -- -0.625

s_sev_seg_5_data <= "01000";

s_sev_seg_4_data <= "10000";

s_sev_seg_3_data <= "00001";

s_sev_seg_2_data <= "10110";

s_sev_seg_1_data <= "10010";

s_sev_seg_0_data <= "10101";

when "11100" => -- -0.500

s_sev_seg_5_data <= "01000";

s_sev_seg_4_data <= "10000";

s_sev_seg_3_data <= "00001";

s_sev_seg_2_data <= "10101";

s_sev_seg_1_data <= "10000";

s_sev_seg_0_data <= "10000";

when "11101" =>  -- -0.375

s_sev_seg_5_data <= "01000";

s_sev_seg_4_data <= "10000";

s_sev_seg_3_data <= "00001";

s_sev_seg_2_data <= "10011";

s_sev_seg_1_data <= "10111";

s_sev_seg_0_data <= "10101";

when "11110" => -- -0.250

s_sev_seg_5_data <= "01000";

s_sev_seg_4_data <= "10000";

s_sev_seg_3_data <= "00001";

s_sev_seg_2_data <= "10010";

s_sev_seg_1_data <= "10101";

s_sev_seg_0_data <= "10000";

when "11111" => -- -0.125

s_sev_seg_5_data <= "01000";

s_sev_seg_4_data <= "10000";

s_sev_seg_3_data <= "00001";

s_sev_seg_2_data <= "10001";

s_sev_seg_1_data <= "10010";

s_sev_seg_0_data <= "10101";

when "00000" => -- 0.000

s_sev_seg_5_data <= "00000";

s_sev_seg_4_data <= "10000";

s_sev_seg_3_data <= "00001";

s_sev_seg_2_data <= "10000";

s_sev_seg_1_data <= "10000";

s_sev_seg_0_data <= "10000";

when "00001" => -- 0.125

s_sev_seg_5_data <= "00000";

s_sev_seg_4_data <= "10000";

s_sev_seg_3_data <= "00001";

s_sev_seg_2_data <= "10001";

s_sev_seg_1_data <= "10010";

s_sev_seg_0_data <= "10101";

when "00010" => -- 0.250

s_sev_seg_5_data <= "00000";

s_sev_seg_4_data <= "10000";

s_sev_seg_3_data <= "00001";

s_sev_seg_2_data <= "10010";

s_sev_seg_1_data <= "10101";

s_sev_seg_0_data <= "10000";

when "00011" => -- 0.375

s_sev_seg_5_data <= "00000";

s_sev_seg_4_data <= "10000";

s_sev_seg_3_data <= "00001";

s_sev_seg_2_data <= "10011";

s_sev_seg_1_data <= "10111";

s_sev_seg_0_data <= "10101";

when "00100" =>  -- 0.500

s_sev_seg_5_data <= "00000";

s_sev_seg_4_data <= "10000";

s_sev_seg_3_data <= "00001";

s_sev_seg_2_data <= "10101";

s_sev_seg_1_data <= "10000";

s_sev_seg_0_data <= "10000";

when "00101" => -- 0.625

s_sev_seg_5_data <= "00000";

s_sev_seg_4_data <= "10000";

s_sev_seg_3_data <= "00001";

s_sev_seg_2_data <= "10110";

s_sev_seg_1_data <= "10010";

s_sev_seg_0_data <= "10101";

when "00110" => -- 0.750

s_sev_seg_5_data <= "00000";

s_sev_seg_4_data <= "10000";

s_sev_seg_3_data <= "00001";

s_sev_seg_2_data <= "10111";

s_sev_seg_1_data <= "10101";

s_sev_seg_0_data <= "10000";

when "00111" => -- 0.875

s_sev_seg_5_data <= "00000";

s_sev_seg_4_data <= "10000";

s_sev_seg_3_data <= "00001";

s_sev_seg_2_data <= "11000";

s_sev_seg_1_data <= "10111";

s_sev_seg_0_data <= "10101";

when "01000" => -- 1.000

s_sev_seg_5_data <= "00000";

s_sev_seg_4_data <= "10001";

s_sev_seg_3_data <= "00001";

s_sev_seg_2_data <= "10000";

s_sev_seg_1_data <= "10000";

s_sev_seg_0_data <= "10000";

when others =>

null;

end case;

end if;

end if;

end process;

end architecture;

seven_seg.vhd

library ieee;

use ieee.std_logic_1164.all;

entity seven_seg is

port (

data  : in std_logic_vector(4 downto 0);

led   : out std_logic_vector(6 downto 0)

);

end entity;

architecture rtl of seven_seg is

begin

led <=  "1000000" when (data = "10000") else -- "0"

"1111001" when (data = "10001") else -- "1"

"0100100" when (data = "10010") else -- "2"

"0110000" when (data = "10011") else -- "3"

"0011001" when (data = "10100") else -- "4"

"0010010" when (data = "10101") else -- "5"

"0000010" when (data = "10110") else -- "6"

"1111000" when (data = "10111") else -- "7"

"0000000" when (data = "11000") else -- "8"

"0010000" when (data = "11001") else -- "9"

"0001000" when (data = "11010") else -- "A"

"0000011" when (data = "11011") else -- "b"

"0100111" when (data = "11100") else -- "c"

"0100001" when (data = "11101") else -- "d"

"0000110" when (data = "11110") else -- "E"

"0001110" when (data = "11111") else -- "F"

"1111111" when (data = "00000") else -- "blank"

"1110111" when (data = "00001") else -- "_"

"1000110" when (data = "00010") else -- "C"

"0100011" when (data = "00011") else -- "o"

"0010010" when (data = "00100") else -- "S"

"1111011" when (data = "00101") else -- "i"

"0101011" when (data = "00110") else -- "n"

"0001100" when (data = "00111") else -- "P"

"0111111" when (data = "01000") else -- "-"

"0101111" when (data = "01001") else -- "r"

"0000111" when (data = "01010") else -- "t"

"1111111";

end architecture;

Sincos_top.vhd

library IEEE;

use IEEE.std_logic_1164.all;

entity sincos_top is

port (

clk: in std_logic;-- Input clock

areset : in std_logic;

a: in std_logic_vector(9 downto 0) := (others => '0');

sev_seg_0_o : out std_logic_vector(6 downto 0);

sev_seg_1_o : out std_logic_vector(6 downto 0);

sev_seg_2_o : out std_logic_vector(6 downto 0);

sev_seg_3_o : out std_logic_vector(6 downto 0);

sev_seg_4_o : out std_logic_vector(6 downto 0);

sev_seg_5_o : out std_logic_vector(6 downto 0);

cd_button   : in std_logic

);

end;

architecture arch of sincos_top is

component sincos is

port (

clk    : in  std_logic                    := 'X';             -- clk

areset : in  std_logic                    := 'X';             -- reset

a      : in  std_logic_vector(9 downto 0) := (others => 'X'); -- a

c      : out std_logic_vector(4 downto 0);                    -- c

s      : out std_logic_vector(4 downto 0)                     -- s

);

end component sincos;

signal cordic_data_s : std_logic_vector(4 downto 0);

signal cordic_data_c : std_logic_vector(4 downto 0);

begin

u0 : component sincos

port map (

clk    => clk,    --    clk.clk

areset => not areset, -- areset.reset

a      => a,      --      a.a

c      => cordic_data_c,      --      c.c

s      => cordic_data_s       --      s.s

);

sincos_disp_top_inst : entity work.sincos_disp_top

port map (

clk => clk,

reset => not areset,

cd_button => cd_button,

cordic_data_s => cordic_data_s,

cordic_data_c => cordic_data_c,

sev_seg_0_o => sev_seg_0_o,

sev_seg_1_o => sev_seg_1_o,

sev_seg_2_o => sev_seg_2_o,

sev_seg_3_o => sev_seg_3_o,

sev_seg_4_o => sev_seg_4_o,

sev_seg_5_o => sev_seg_5_o

);

end;

sincos_tb.vhd

library ieee;

use ieee.std_logic_1164.all;

-- use ieee.std_logic_unsigned.all;

use ieee.numeric_std.all;

entity sincos_top_tb is

end;

architecture bench of sincos_top_tb is

-- Clock period

constant clk_period : time := 5 ns;

constant pos_pi : signed := "0110010010";

constant neg_pi : signed := "1001101110";

constant phase_inc : signed := "00100";

-- Generics

-- Ports

signal clk : std_logic := '0';

signal areset : std_logic := '0';

signal a : std_logic_vector(9 downto 0) := (others => '1');

signal c : std_logic_vector(4 downto 0);

signal s : std_logic_vector(4 downto 0);

signal finished : boolean := false;

signal cd_button   : std_logic := '0';

begin

sincos_inst : entity work.sincos_top

port map (

clk => clk,

areset => areset,

a => a,

c => c,

s => s,

cd_button  =>  cd_button

);

process is

begin

clk <= '1';

wait for clk_period;

clk <= '0';

wait for clk_period;

if finished then

wait;

end if;

end process;

process is

variable cnt : integer := 0;

begin

if ((phase_inc + signed(a) ) > pos_pi) then

a <= std_logic_vector( neg_pi) ;

else

a <= std_logic_vector( phase_inc + signed(a));

end if;

wait for clk_period * 2;

if (cnt > 2000) then

finished <= true;

wait for clk_period;

wait for clk_period;

wait for clk_period;

else

cnt := cnt + 1;

end if;

end process;

end;