Due Date: Sunday November  17, 11:59pm– Submit this editable Word document NOT pdf

0

12

EEDG/CE 6370

Design and Analysis or Reconfigurable Systems

Homework 8 – Hard Processor System

Name: 	Mark Ivey					Email: mxi240020@utdallas.edu

Part I - Linux

Install any of the Linux versions on the SD card and show that it is working. Include picture.

Marks

2

Part II – Simple program

Write a simple “Hello UTD” Program” and compile and run it on the HPS of the FPGA. Report the size of the compiled file. Include a video of the compilation process and working design showing the program runs on the FPGA.

Marks

4

https://www.youtube.com/watch?v=6pjJhUETS_I

Part III – Controlling Peripherals through HPS

- As you can see from the picture below, the HPS in the Terasic board is ‘connected’ to different peripherals:

Figure 1 Cyclone V SoC in Terasic DE1-SoC board overview

Some of these peripherals include an HPS user button and a HPS user LED. Compile the hps_gpio/main.c program and annotate next to each line what it does through C/C++ comments. Include the size of the compiled program too. Create a YourTube video showing that it works.

Marks

6

Size of original a.out program: 8332 bytes

https://www.youtube.com/watch?v=SwKurNImsB8

Main.c File

#define soc_cv_av

#include <stdio.h>

#include <unistd.h>

#include <fcntl.h>

#include <sys/mman.h>

#include "hwlib.h"

#include "socal/socal.h"

#include "socal/hps.h"

#include "socal/alt_gpio.h"

#define HW_REGS_BASE ( ALT_STM_OFST )

#define HW_REGS_SPAN ( 0x04000000 )

#define HW_REGS_MASK ( HW_REGS_SPAN - 1 )

#define USER_IO_DIR     (0x01000000)

#define BIT_LED         (0x01000000)

#define BUTTON_MASK     (0x02000000)

int main(int argc, char **argv) {

void *virtual_base;

int fd;

uint32_t  scan_input;

int i;

// map the address space for the LED registers into user space so we can interact with them.

// we'll actually map in the entire CSR span of the HPS since we want to access various registers within that span

if( ( fd = open( "/dev/mem", ( O_RDWR | O_SYNC ) ) ) == -1 ) {

// this allows us to read and write directly to memory;

// Requires elevated privileges

printf( "ERROR: could not open \"/dev/mem\"...\n" );

return( 1 );

}

virtual_base = mmap( NULL, HW_REGS_SPAN, ( PROT_READ | PROT_WRITE ), MAP_SHARED, fd, HW_REGS_BASE );

//declare the base address

printf("virtual_base address: %p\n", virtual_base); // returns 0xb2e2f000

if( virtual_base == MAP_FAILED ) {

printf( "ERROR: mmap() failed...\n" );

close( fd );

return( 1 );

}

// initialize the pio controller

// led: set the direction of the HPS GPIO1 bits attached to LEDs to output

//alt_setbits_word

alt_setbits_word( ( virtual_base + ( ( uint32_t )( ALT_GPIO1_SWPORTA_DDR_ADDR ) & ( uint32_t )( HW_REGS_MASK ) ) ), USER_IO_DIR );

printf("Offset added to virtual_base to enable LED GPIO: %p\n", (void *)((uint32_t)(ALT_GPIO1_SWPORTA_DDR_ADDR) & (uint32_t)(HW_REGS_MASK)));

// this offset for LED GPIO is 0x3709004

//printf("virtual_base + offset address: %p\n", ( virtual_base + ( ( uint32_t )( ALT_GPIO1_SWPORTA_DDR_ADDR ) & ( uint32_t )( HW_REGS_MASK ) ) ));

printf("led test\r\n");

printf("the led flash 2 times\r\n");

printf("Offset added to virtual_base to blink LED: %p\n", (void *)( ( uint32_t )( ALT_GPIO1_SWPORTA_DR_ADDR ) & ( uint32_t )( HW_REGS_MASK ) ) );

// this offset is 0x3709000

for(i=0;i<2;i++)

{

alt_setbits_word( ( virtual_base + ( ( uint32_t )( ALT_GPIO1_SWPORTA_DR_ADDR ) & ( uint32_t )( HW_REGS_MASK ) ) ), BIT_LED );

usleep(500*1000);

//alt_write_word(dest, alt_read_word(dest) | (bits))

// Reads the data in the base + offset address

// modifies the data using OR BIT_LED with the value retrieved from the address (0xb2e2f000 + 0x3709000)

// writes the value to the base + offset address

alt_clrbits_word( ( virtual_base + ( ( uint32_t )( ALT_GPIO1_SWPORTA_DR_ADDR ) & ( uint32_t )( HW_REGS_MASK ) ) ), BIT_LED );

usleep(500*1000);

//alt_write_word(dest, alt_read_word(dest) & ~(bits))

// Reads the data in the base + offset address

// modifies the data using INVERT then AND BIT_LED with the value retrieved from the address (0xb2e2f000 + 0x3709000)

// writes the value to the base + offset address

}

printf("user key test \r\n");

printf("press key to control led\r\n");

printf("Offset added to virtual_base for key press: %p\n", (void *)(( uint32_t )(  ALT_GPIO1_EXT_PORTA_ADDR ) & ( uint32_t )( HW_REGS_MASK )));

while(1){ // start superloop

scan_input = alt_read_word( ( virtual_base + ( ( uint32_t )(  ALT_GPIO1_EXT_PORTA_ADDR ) & ( uint32_t )( HW_REGS_MASK ) ) ) );

// poll offset address for the button...

//usleep(1000*1000);

if(~scan_input&BUTTON_MASK) //... and check if it is being pressed

alt_setbits_word( ( virtual_base + ( ( uint32_t )( ALT_GPIO1_SWPORTA_DR_ADDR ) & ( uint32_t )( HW_REGS_MASK ) ) ), BIT_LED );

// if pressed, turn the LED on

else    alt_clrbits_word( ( virtual_base + ( ( uint32_t )( ALT_GPIO1_SWPORTA_DR_ADDR ) & ( uint32_t )( HW_REGS_MASK ) ) ), BIT_LED );

// if not pressed, turn the LED off

}

// clean up our memory mapping and exit

if( munmap( virtual_base, HW_REGS_SPAN ) != 0 ) {

printf( "ERROR: munmap() failed...\n" );

close( fd );

return( 1 );

} // if we never enter the superloop (ie, no root acesss to read/write to /dev/mem/) then close fd and exit

close( fd );

return( 0 );

}